UNIVERSITY OF CALIFORNIA

College of Engineering

Department of Electrical Engineering and Computer Sciences

EE 130/230A	Prof. Liu

Fall 2013

Homework Assignment #9

Due at the beginning of class on Thursday, 10/31/13

# Problem 1: MOS Capacitor - Areal Charge Density

Consider an n+ poly-Si-gated capacitor (M = 4.05 eV) with oxide thickness xo = 3 nm and p-type Si substrate doping NA = 1017 cm-3, maintained at T = 300K.

- Calculate the flatband voltage, VFB.

- Calculate the threshold voltage, VT.

- Plot the value of the total areal charge density (in C/cm2) in the Si as a function of gate voltage VG, for each of the regions of operation (accumulation, depletion and inversion).  Indicate the numerical values of areal charge density in the Si at VG = VT and VG = VT + 1 Volt.

Problem 2: MOS Threshold Voltage

Consider an NMOS capacitor with oxide thickness xo = 3 nm, maintained at T = 300K.

- Generate a plot of the threshold voltage (on a linear scale) vs. Si dopant concentration (on a log scale), for p-type substrate doping NA ranging from 1015 cm-3 to 1018 cm-3, for two cases:

  - Gate material is n+ poly-Si (M = 4.05 eV)

  - Gate material is a “mid-gap” metal (M = 4.6 eV)

- From your plot in (a), estimate the value of NA corresponding to a threshold voltage of 0.5 V, for each of the two cases.

- Qualitatively, how would your answers to part (b) change if the oxide thickness xo were to be decreased (e.g. to 2 nm)?

Problem 3: MOS C-V characteristic

The gate capacitance vs. gate voltage characteristic of a p+ poly-Si gated MOS capacitor (M = 5.17 eV) of area 110-4 cm2, is as shown:

- Is the semiconductor (silicon) substrate doped n-type or p-type?  Explain briefly.

- Is the measurement frequency low or high?  Explain briefly.

- What is the thickness of the gate oxide (SiO2), xo?

- Estimate the semiconductor doping concentration.  (Note: You should be able to estimate the flatband voltage, VFB, from the C-V plot above.)

- Based on your answers to (c) and (d), calculate the value of the minimum capacitance, Cmin.